TRACK B — STUDENT LEARNING MODULES

Module B2: Vision, Uncertainty & Confidence Scoring

Audience: IE / MfgE students — lab access required   |   Estimated time: 90 minutes

Cal Poly SLO  ·  Industrial & Manufacturing Engineering  ·  Digital Twin Project

# Introduction

Automated systems often fail not because they encounter something unmanageable, but because they do not know they cannot manage it. A robot that misclassifies an object with wrong certainty causes more damage than one that pauses to ask. This module builds your understanding of how the vision system quantifies its own uncertainty.

Learning objectives - Explain how HSV color extraction and K-means clustering produce a classification - Interpret a confidence score and explain what drives it higher or lower - Identify the physical conditions that push confidence below the intervention threshold - Predict, before running a trial, whether a scenario will trigger human intervention - Explain the tradeoff between setting the threshold higher vs. lower

# How the Vision System Works

## Stage 1: HSV color extraction

Each pixel is converted from RGB to HSV. HSV separates color identity (hue) from lighting effects (value), making classification more robust to ambient light variation. Significant lighting changes — a shadow, a different lamp — can shift an object's HSV values outside its trained range. This is the most common cause of sudden confidence degradation.

## Stage 2: K-means classification

Each detected object is characterized by its average HSV values and shape geometry. K-means assigns it to the nearest cluster centroid. Confidence score: 1 minus the normalized distance from the observation to its nearest centroid. An observation on a centroid scores 1.0; one equidistant between two centroids scores near 0.5.

## Stage 3: Threshold comparison

The confidence score is compared to the calibrated threshold (0.607). Above: robot proceeds autonomously. Below: system pauses and routes to an operator. The threshold was calibrated empirically for this specific task and objects.

Figure 1. Confidence score distributions showing the intervention zone and autonomous execution zone

# Lab Activity: Inducing and Observing Low Confidence

## Pre-lab prediction (complete before the lab)

| Condition | Your prediction | Reasoning |
| --- | --- | --- |
| Normal conditions (baseline) |  |  |
| Object partially occluded |  |  |
| Shadow cast across workspace |  |  |
| Two objects of similar color |  |  |
| Object rotated 90° from training |  |  |

## Lab observations (complete during lab)

| Condition | Conf. score | Prediction correct? | Notes on camera view |
| --- | --- | --- | --- |
| Baseline |  |  |  |
| Occlusion |  |  |  |
| Shadow |  |  |  |
| Similar color |  |  |  |
| Rotated 90° |  |  |  |

# Threshold Tradeoff Analysis

| Threshold raised to 0.80 - More human interventions: every borderline case is reviewed - Higher autonomous accuracy rate - Longer cycle times overall - Use when cost of errors is high (e.g., surgical assembly) | Threshold lowered to 0.40 - Fewer interventions: more autonomous decisions - Lower autonomous accuracy rate - Faster cycle times overall - Use when cost of delay exceeds cost of occasional errors |
| --- | --- |

# Discussion Questions

For small group or written reflection - You ran the occlusion condition and confidence dropped to 0.31. Should you raise or lower the threshold? Explain why neither direction is automatically correct. - The system correctly classified an object with a confidence score of 0.42. A student argues the threshold is too high. How do you respond? - In Zehrung's DT vision, operators observe from 3,000 miles away. What challenges does remote monitoring create for confidence management not present in the lab? - Name one application where you would set the threshold much higher than 0.607 and one much lower. Justify both.

References: Lenz et al. (2015); Jasim & Plapper (2014); Collet et al. (2011); Rasheed et al. (2020)