TRACK B — STUDENT LEARNING MODULES

Module B3: Human-in-the-Loop Decision Making

Audience: IE / MfgE students — lab access required   |   Estimated time: 90–120 minutes (CORE MODULE)

Cal Poly SLO  ·  Industrial & Manufacturing Engineering  ·  Digital Twin Project

This is the core module. The competency practiced here — knowing when to trust automation and intervening appropriately — is the central Industry 5.0 skill identified as absent from DT education by Hazrat et al. (2023).

# Introduction

You are about to become part of the robot's decision-making architecture. When vision confidence drops below 0.607, the robot stops and asks for your input. Your response directly controls what happens next. This is not a simulation — it is the actual system. Your decisions will be logged, and you will analyze them afterward.

Learning objectives - Respond accurately and consistently to confidence-triggered intervention requests - Compare your decision accuracy to the autonomous system at the same confidence levels - Identify the types of cases where human judgment adds the most value - Explain shared autonomy and where it applies in Industry 5.0 manufacturing - Articulate what the logged data from your interventions could be used for beyond this exercise

# Background: Shared Autonomy

Shared autonomy is the formal term for systems where control is dynamically split between a human and an autonomous agent based on context. Dragan & Srinivasa (2013) formalize this as a policy-blending framework: autonomous policy and human policy are combined, with the human's weight increasing as confidence decreases. In this lab the blending is a hard threshold — a deliberate simplification that makes the human role unambiguous and the data easy to analyze.

Broad et al. (2017) showed that systems can learn from human corrections: the more interventions are logged, the better the autonomous system can model what a human would do, requesting intervention less often over time. Your corrections today are training data for a future version of this system.

Figure 1. HitL decision architecture — you are in the "Human Input" box

# Part 1: Baseline Human Performance (20 trials)

## What you will see when an intervention is triggered

- The robot arm stops mid-operation — expected and intended

- The operator panel displays the camera view with the object highlighted

- The confidence score is shown (below 0.607) along with the system's best guess

- You may confirm, override, or mark as unclear

- A timer records your response time

| # | Conf. | System guess | Your response | Agree? | Time (s) |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |
| 11 |  |  |  |  |  |
| 12 |  |  |  |  |  |
| 13 |  |  |  |  |  |
| 14 |  |  |  |  |  |
| 15 |  |  |  |  |  |
| 16 |  |  |  |  |  |
| 17 |  |  |  |  |  |
| 18 |  |  |  |  |  |
| 19 |  |  |  |  |  |
| 20 |  |  |  |  |  |

# Part 2: Degraded Conditions (10 trials)

Condition introduced: _____________________________________________

| # | Conf. | System guess | Your response | Agree? | Time (s) |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

# Post-Lab Analysis

- Accuracy rate Part 1 (%): ____

- Accuracy rate Part 2 (%): ____

- Agreement rate with system guess (%): ____

- Average response time Part 1 (s): ____

- Average response time Part 2 (s): ____

The autonomous system's accuracy at confidence levels below 0.607 is approximately 61% — it is wrong more than one time in three. Your task is to beat that number.

Questions for your post-lab writeup - What was your accuracy rate in Part 1? Did you outperform the autonomous system's 61% at that confidence level? - Did accuracy or response time change between Part 1 and Part 2? What does this reveal about human vs. machine performance under uncertainty? - In which cases did you disagree with the system's guess? What did you see in the camera view that the confidence score did not capture? - Broad et al. (2017) argue logged human corrections can train the autonomous system to improve. Based on your interventions, what would the system need to learn? - At what production volume would you start questioning whether HitL review remains economically justified?

References: Hazrat et al. (2023); Dragan & Srinivasa (2013); Broad et al. (2017); Amershi et al. (2014); Villani et al. (2018)